# WHAMO

Network Designer

User Manual

Hydraulic Transient Analysis · Water Hammer & Mass Oscillation

| DOCUMENT INFORMATION | DETAIL |
| --- | --- |
| Prepared for submission to | Ministry of Jal Shakti, Government of India |
| Software | WHAMO Network Designer, version 1.0 (as displayed in the application) |
| Manual revision | 1.0 |
| Date | 28 October 2026 |
| Prepared by | [Project team / intern names] |
| Institution / host organization | [Enter institution or organization] |

Document status: Project user manual prepared for submission and technical review. It is not an official Ministry publication, an approval of the software, or an engineering design certification. Complete the author and organization fields and verify the deployed application version before final submission.

# Contents

1.  About this manual and the software

2.  Access and account roles

3.  Start, open, and save a project

4.  Designer screen and network workflow

5.  Network elements and their principal data

6.  Units, conduits, and shape calculations

7.  Turbine setup

8.  Simulation parameters and output requests

9.  Generate, edit, and import WHAMO files

10.  Visualize simulation results

11.  Flex Table and Excel exchange

12.  Settings and user management

13.  Validation, review, and troubleshooting

14.  Glossary

## 1. About this manual and the software

WHAMO Network Designer is a visual application for creating hydraulic network models, entering model properties and schedules, preparing WHAMO .INP input files, running the configured WHAMO engine where available, and viewing simulation .OUT results.

The application provides a graphical network canvas, property panels, unit selection, project saving and loading, tabular editing, file import/export, validation messages, and profile/history visualization. The available actions may depend on the signed-in user's role and section permissions.

### Intended audience

This guide is for users who have been issued an application account and need to create, review, exchange, or visualize WHAMO network models. It assumes that the user has access to the engineering data and WHAMO version approved for the study.

### Important engineering limitation

The Designer helps users prepare and inspect model files. Its validation checks are software checks; they do not establish that the network topology, measurements, material assumptions, boundary conditions, turbine characteristics, operating scenarios, or results are physically correct or safe. Use project-approved data, inspect generated files and WHAMO diagnostics, and obtain review by a qualified person before relying on results.

### Separate applications

The main WHAMO Designer includes an embedded visualization view. A separate WHAMO Visualization web application is also provided in this project. Both can read supported WHAMO .OUT content; the standalone viewer is useful when an output file is being reviewed independently of an open Designer network.

## 2. Access and account roles

### Sign in

1.  Open the application address supplied by the project administrator.

2.  Enter the email address and password assigned to your account.

3.  Select Sign In.

4.  After successful authentication, the Designer opens. If sign-in fails, confirm the account details with the administrator rather than repeatedly guessing passwords.

The login page may display a demonstration account for demonstration/testing. Do not use demonstration credentials for operational work or enter them into a report, screenshot, or shared project file.

### Roles and section permissions

The application distinguishes Master Admin and Normal User accounts. A Master Admin can manage user accounts and assign access to application sections. A Normal User can use the sections enabled for that account. If a required tool is missing, ask the administrator to confirm the account's permissions.

Do not share accounts. Sign out from the user menu when finished, especially on shared computers.

## 3. Start, open, and save a project

### Start a new project

1.  On the start screen, choose New Project, or choose New in the title bar.

2.  Enter a meaningful project name in the name field.

3.  Add the network elements and connections required for the model.

4.  Select Save to save the project to your account.

Use a name that identifies the study and revision without including confidential information.

### Open a saved project or local file

•  Select Open to choose a saved project from the project list.

•  To load a local project file, use Open and select a .json project file or a supported .INP file.

•  To import an input file into the current Designer workflow, select From .INP, choose the file, review the parsed network summary, and confirm the import.

After loading, check the project name, network diagram, element properties, schedules, simulation settings, unit system, and output requests. An import should always be reviewed before editing or generating a new file.

### Save and create a separate copy

•  Save updates the current project in your account. The first save creates the account copy.

•  Save As downloads a separate project copy in JSON format. Keep this file as a local backup or revision copy.

•  Undo and Redo reverse or restore recent edits. Save before substantial changes, bulk imports, or experiments.

Saving a project and generating an .INP are separate actions. Save the project after editing; generate the input file when you are ready to prepare a WHAMO run.

## 4. Designer screen and network workflow

### Main screen areas

| Area | Purpose |
| --- | --- |
| Title bar | Start/open/save projects, undo/redo, arrange the diagram, import .INP, open Help, set the project name, choose units, and access account controls. |
| Insert ribbon | Add supported network elements and choose the turbine connection tool. |
| Tools ribbon | Open the diagram view, node selection, Flex Table, visualization, or 3D view. |
| Generate ribbon | Start .INP export or .OUT generation. Each action opens validation and generation settings as applicable. |
| Network canvas | Display and connect the model. Diagram position is for readability and does not define hydraulic correctness by itself. |
| Properties panel | Edit the selected node or connection. Fields and units vary by element. |

### Add and select elements

1.  Select an element in the Insert ribbon: Reservoir, Node, Junction, Surge Tank, Flow BC, or Turbine.

2.  Select the new item on the canvas to open its properties.

3.  Enter a unique, meaningful label and the required model values.

4.  Save the project after completing the element.

Conduits are network connections between elements. Select or create a connection using the connection controls, then select it to edit its properties. Where offered, the connection's Type can be set to Conduit or Dummy Pipe. The Turbine ribbon has a split control for placing a turbine as an element or activating turbine-connection mode.

Some imported projects may retain older data fields that are not available for new insertion. Review the import summary and the imported properties before generating a new file.

### Connect and arrange the network

1.  Connect the intended upstream and downstream elements using their connection handles.

2.  Check the direction arrow, labels, and endpoints on every connection.

3.  Use Arrange to improve diagram layout when useful.

4.  Confirm the actual hydraulic connectivity independently of the visual layout.

Use nodes and junctions to express network topology. A surge tank is a side-branch element and should connect to one appropriate node/junction; it should not be inserted inline as a through-flow conduit.

### Edit and commit properties

1.  Select an element or conduit on the canvas.

2.  Review the fields shown in the Properties panel and their displayed units.

3.  Enter the approved values. Some panels hold changes locally until Save is selected.

4.  Confirm the displayed values and save the project.

When a field is automatically filled from a selected material or calculated from another field, verify its source inputs. Where the interface marks a value as editable, confirm the engineering basis before overriding it.

## 5. Network elements and their principal data

The precise fields shown depend on the selected element and project configuration. Use approved study data; the descriptions below explain the role of the main categories, not the engineering method for selecting their values.

| Element | General role | Data to review |
| --- | --- | --- |
| Reservoir | Boundary representing a reservoir/source condition. | Label and reservoir elevation; confirm the boundary value and its datum. |
| Node | Network point used to connect conduits and report hydraulic quantities. | Label/number, elevation and any other displayed hydraulic or reporting fields. |
| Junction | Connection/split point for branches in the model. | Label/number, connections, and any displayed junction values. |
| Flow BC | Flow boundary condition driven by a flow schedule. | Label, schedule number, and the schedule's time/flow pairs. |
| Conduit / Dummy Pipe | Connection type selected in the connection's properties. | Label, connection type, length, diameter/shape, celerity, friction, material values, and any other displayed properties. |
| Surge Tank | Side-branch storage element used in the transient network. | Top and bottom elevations, cross-section mode, diameter or SHAPE data, and displayed hydraulic properties. |
| Turbine | Turbine component with characteristic data and an operating mode. | TCHAR type, synchronous speed, inertia, diameter, friction, windage, operation mode, and applicable schedule fields. |

### Schedules

Schedules are time/value pairs referenced by an element. Check that each element points to the intended schedule number and that pair order, units, values, and time range match the study. Schedule definitions can be reviewed in the properties panel or supported table editors.

## 6. Units, conduits, and shape calculations

### SI and FPS units

The title-bar Units control switches the global Designer unit system between SI and FPS. A selected element can also display its own unit control. An element that is explicitly locked to a unit system may not follow the global switch until it is unlocked.

Before entering or importing values:

1.  Select the intended unit system.

2.  Check the unit labels beside the fields.

3.  Do not copy a value from a different unit system without converting it.

4.  After switching units, review critical values and units before saving or generating a file.

The visualization's SI/FPS control changes displayed result units; it does not rewrite the source .OUT file.

### Conduit diameter and shape-equivalent diameter

The conduit Diameter Shape Type selector defaults to Circular. The other named profiles are Horse Shoe, Modified Horse Shoe, and D-Shaped. For a non-circular profile, the entered diameter is treated as the shape's reference dimension, and the displayed equivalent diameter is calculated from cross-sectional area:

A_shape = k × D² A_circle = 0.785 × D_eq² D_eq = √(k / 0.785) × D

The current coefficients are:

| Shape | Area coefficient k | Approximate D_eq / D |
| --- | --- | --- |
| Circular | 0.785 | 1.000 |
| Horse Shoe | 0.829 | 1.028 |
| Modified Horse Shoe | 0.893 | 1.067 |
| D-Shaped | 0.813 | 1.018 |

The resulting D_eq is used as the conduit diameter in the generated .INP. The display shows the calculation and the resulting value. For example, a 10 m Horse Shoe reference diameter gives D_eq ≈ 10.28 m.

This is an area-equivalence conversion to a circular diameter for WHAMO input. It does not mean the application has performed a full non-circular hydraulic analysis. Confirm that this approximation is suitable for the study.

### Pipe material

The current material selector offers Steel and Concrete. Selecting a material can automatically fill Manning's n and Young's modulus E; values remain subject to engineering review. The configured Manning's n values are 0.011 for Steel and 0.013 for Concrete. Use the expandable material-properties display to inspect the selected material's listed values. The Apply to all conduits in network option applies the selected material across the network; use it only when that is intended.

### Surge tank cross-section

The surge tank keeps a separate Enable SHAPE toggle:

•  SHAPE disabled: enter a diameter. The Diameter Shape Type selector can apply the same named-shape area-equivalent diameter calculation described above; the displayed equivalent diameter is the diameter used in the .INP.

•  SHAPE enabled: provide SHAPE table pairs. Select a standard shape to calculate its area from A = k × D² for each entered D value. For Custom, provide the applicable E and A values directly.

Here, E identifies the elevation for a shape pair, D is the shape dimension used by the selected standard profile, and A is the corresponding cross-sectional area. Use consistent units and elevation references for every pair. The program may calculate a perimeter for named profiles, but the SHAPE pairs and geometry should still be reviewed against the project's source data.

## 7. Turbine setup

Select a turbine to open its Properties panel. Turbine behavior depends on the selected characteristic set and operation data; use project-approved manufacturer or study inputs.

### Turbine characteristics (TCHAR)

Each turbine references a TCHAR Type. The characteristic data describe turbine response across specified heads and gate openings. The software supports editing the characteristic tables and, where available, using a built-in curve generator. The generated curve is only an aid; it is not a substitute for verified manufacturer data.

The characteristic data include flow (Q) and efficiency values associated with the characteristic points. Confirm the table dimensions, head/gate alignment, units, and source before using it in a study. Do not assume an automatically generated efficiency or flow value is manufacturer-approved.

### Built-in performance-curve generator and Q/efficiency matrices

The optional Performance Curves — TCHAR Generator fills a turbine characteristic table from the software's built-in hill-chart library. It is an estimation aid, not a replacement for verified manufacturer or project data.

1.  Select a specific-speed (Ns) library group. The current choices are Ns 30, Ns 45, and Ns 60; the panel shows each group's stated head range and a design-head-based recommendation.

2.  Enter Design Head (H), Design Flow (Q), and Design Efficiency (η). Efficiency is entered as a decimal from 0 to 1 (for example, 0.90).

3.  Choose Head Steps from 2 to 20 and select one or more gate-opening percentages. The built-in library provides gates from 20% through 100%.

4.  Review the curve preview, then choose Generate TCHAR From Curves. If the selected TCHAR table already contains data, confirm the overwrite. The generated table can then be edited directly.

The initial form selects Ns 30, gates 20/40/60/80/100, design flow 100, design efficiency 0.90, and 5 head steps. Design head starts with the turbine's existing value when available, otherwise 50. These are interface defaults, not recommended design values. Treat the Ns recommendation as a convenience hint only; confirm the selected library group and units against the approved turbine data.

The software samples normalized head values across the selected library range and scales them to the design head, rounding scaled head values to three decimal places. For each selected gate and sampled head, it linearly interpolates the library's flow and torque values. If a sampled head is outside the available points for a selected gate, the implementation uses that curve's nearest endpoint rather than extrapolating. The generated matrices use:

H_scaled = H_normalized × H_design / 100 Q_matrix = Q_library_interpolated × Q_design / 100 η_raw = [T_library_interpolated / (H_normalized × Q_library_interpolated)] × (η_design × 100) η_matrix = min(0.98, max(0, η_raw))

The generator uses zero efficiency when normalized head or interpolated flow is at or below 1, rounds flow to three decimal places and efficiency to four, and caps the stored efficiency value at 0.98. Each matrix row corresponds to a selected gate; each column corresponds to a generated head value. These are the program's implemented calculations—not a guarantee that the built-in curve is suitable for a specific turbine. Verify units, curve provenance, matrix values, and the resulting WHAMO input with the project engineer and the WHAMO version in use. Do not reinterpret 0.98 as 98 unless the required WHAMO format explicitly calls for that conversion.

### Operation modes

The current operation-mode choices in the Designer are:

| Mode | User-level description |
| --- | --- |
| GENERATE | Represents the generating operating case. A V-schedule can define gate opening over time. |
| OFF | Represents the turbine-off case. |
| REJECT | Represents a load-rejection case. The panel exposes TOFF and a V-schedule. |
| STARTUP | Represents a startup case. A V-schedule can define gate opening over time. |

These are the mode labels exposed by this software; they are not a substitute for checking the syntax and interpretation supported by the WHAMO executable/version used for the analysis. Check WHAMO run diagnostics and the relevant WHAMO documentation before relying on a mode in a formal study.

### Turbine fields and schedules

•  TCHAR Type: selects the turbine characteristic dataset.

•  Synchronous Speed (SYNCSPD): turbine/generator synchronous rotational speed, in RPM.

•  WR²: turbine-generator moment of inertia input as represented by the application.

•  Diameter, Friction, Windage: turbine parameters used by the generated input.

•  TOFF: rejection-time field shown for REJECT mode; enter a study-approved value in seconds.

•  VSCHEDULE: time (T) and gate opening (G) pairs used for GENERATE, REJECT, and STARTUP modes. The schedule number on the turbine identifies which schedule is used.

For imported files containing a retired/legacy operation mode, the application preserves legacy data for compatibility and marks it as legacy. Choose a current mode to replace it only after confirming the intended WHAMO case.

## 8. Simulation parameters and output requests

Selecting Generate → .INP or Generate → .OUT first runs network validation. When the validation step allows continuation, the Configure Output Requests sidebar opens. It separates Computational Parameters from Output Configuration.

### Computational parameters

Review each time stage:

| Field | Meaning in the interface |
| --- | --- |
| DTCOMP | Computation time step. |
| DTOUT | Interval between recorded output times. |
| TMAX | Simulation end time for that stage. |

Use Add Stage to add a time stage. At least one stage is retained. The panel also contains the Include ACCUTEST in .INP checkbox and the ACCUTEST Mode choices:

| Mode | Label shown in the software |
| --- | --- |
| FULL | High Accuracy |
| PARTIAL | Moderate |
| NONE | No Checking |

These settings should come from the approved analysis plan and the WHAMO version in use; the software does not choose engineering-appropriate values on the user's behalf. The in-app Help also calls out the Courant stability check:

Courant number = celerity × DTCOMP / conduit length ≤ 1.0

Check this condition for the modeled conduits using consistent units. A successful file export does not establish that the chosen time step is appropriate.

### Output configuration

Only HISTORY is shown as the request type. Network elements are not automatically added to the output request list when they are inserted. Requests are created only after a user explicitly adds them in the output panel.

To add one request:

1.  Choose a category under Select Type if you want to filter/group available items.

2.  Choose the specific item under Select Element.

3.  Select the variables to record. Turbine selections offer Q, HEAD, SPEED, and POWER; other selections may offer Q, HEAD, ELEV, VEL, PRESS, and PIEZHEAD.

4.  Select Add Request.

5.  Confirm that the item and variables appear under Current Requests — HISTORY.

To add requests for a group, choose a category, select the relevant variables, and use Add All for .... Select All is a separate explicit action that fills requests for the available network items; inspect the resulting list and remove any request that is not needed. Clear All removes all current requests. These controls do not add requests silently when a network element is created.

## 9. Generate, edit, and import WHAMO files

### Generate an .INP

1.  Enter a specific project name.

2.  Select Generate → .INP.

3.  Review the validation messages. Correct errors and inspect warnings; do not bypass a message without understanding it.

4.  In Configure Output Requests, review Computational Parameters and Output Configuration separately.

5.  Confirm the History request list, then continue to generate the file.

6.  Inspect the .INP preview. Save/download a copy and preserve it with the project revision.

Generated .INP text is an input file for WHAMO. File generation by itself does not run the simulation.

### Edit a generated .INP

The application displays a warning before opening the text editor: editing the .INP changes the text file only. It does not update the values in the network currently open in the Designer.

If the edited file should become the network used for a run:

1.  Save/download the edited .INP.

2.  Import it using From .INP.

3.  Review the parsed network, schedules, parameters, and requests.

4.  Save the imported project as a separate revision if appropriate.

5.  Generate or run the desired output from that imported network.

Keep the original input file. Manually edited text must remain valid for the WHAMO version in use.

### Generate an .OUT

1.  Select Generate → .OUT.

2.  Review validation messages and resolve problems before proceeding.

3.  Check the generation sidebar settings and History requests.

4.  Continue to run the configured WHAMO engine.

5.  Review the returned output and engine diagnostics before interpreting results.

The .OUT operation depends on a WHAMO engine being configured and available to the application. If it is unavailable or the run fails, use the approved WHAMO installation to run the reviewed .INP, then load the resulting .OUT into the visualization tool.

## 10. Visualize simulation results

### Open the visualization view

•  In the Designer, select Tools → Visualization. When a network is open, the application attempts to run a simulation and open its output. If no network is open, the visualization view can be used to load an .OUT file.

•  In the standalone WHAMO Visualization application, select Add .OUT File and choose the output file.

The viewer reads supported summary and history sections from the .OUT. If the file has no readable summary data, the viewer displays an error instead of a graph.

### Profile Graph

The Profile Graph shows summary/profile data across nodes, including maximum and minimum energy elevation and, where applicable, the animated current hydraulic grade line. Use the playback controls to view the animation, the slider to move through time, and the speed controls to adjust playback. Use Expand or Fullscreen where shown for a larger chart. The optional pipe display and labels can be toggled where available.

Expand Raw Simulation Summary Data to inspect values in tabular form. The SI/FPS control in this view applies to the profile chart and its raw summary table together. Verify the displayed axis units before comparing values.

### History Graph

The History Graph is available when the loaded .OUT contains history data. Select one or more nodes, choose the recorded parameter, and use playback controls to review time-dependent behavior. Expand Raw Time History Data to inspect the selected series in a table; large tables may be paginated. The SI/FPS control applies to the history graph and its associated displayed values.

In the Designer's embedded visualization, the SI/FPS control is shared by the Profile Graph and History Graph. In the standalone viewer, verify the unit labels and controls shown in that viewer; its interface may not match the embedded view in every detail. Display-unit controls change presentation only; they do not alter the source .OUT file or recalculate the simulation. Note the source file, WHAMO version, and displayed unit system in any report based on the visualization.

## 11. Flex Table and Excel exchange

### Flex Table

1.  Select Tools → Flex Table.

2.  Choose an element-type tab or the all-elements view.

3.  Review the row labels, field names, and units before editing.

4.  Edit supported cells and commit the edits as shown in the table.

5.  Select a row when you need to locate the corresponding element on the canvas.

6.  Save the project and verify that the canvas properties and table agree.

Some columns are read-only or apply only to specific element types.

### Export or import Excel workbooks

1.  In Flex Table, choose the required element-type tab or all-elements view.

2.  Select the Excel export action and save the .xlsx workbook.

3.  Preserve the workbook's column headings and expected value formats.

4.  Edit only the intended cells. Keep an untouched copy as a backup.

5.  Import the workbook into the corresponding table view.

6.  Review the import summary, skipped rows, and matched elements.

7.  Inspect the affected elements in the Designer and save the project.

Excel import updates matching existing elements; it is not a way to create new network elements. Labels/identifiers are used to match rows, so changing them can prevent a row from matching the intended element.

## 12. Settings and user management

### Personal settings

Open the user menu and choose Settings to access the available profile, security, autosave, and projects settings. Use the security page to change your own password. Follow the application's password requirements and do not include credentials in project documentation.

Autosave settings control automatic account backups for a named project. Autosave is not a replacement for using Save, and it does not replace local .json backups or generated .INP/.OUT files.

### User management (Master Admin only)

1.  Open the user menu and select User Management.

2.  Select Add User to create an account, or the edit control to update a user.

3.  Assign the role and the section access required for that user's duties.

4.  Save the change and verify the displayed account and permissions.

5.  Delete an account only when authorized and after considering whether access or project ownership is affected.

Master Admin accounts have full access. Assign only the minimum sections needed for a user's work. Normal users cannot open the User Management panel.

## 13. Validation, review, and troubleshooting

### Pre-run review checklist

☐  Project name and revision identify the intended study.

☐  Network connectivity and flow direction match the approved schematic.

☐  Element labels and node numbers are unique and valid.

☐  Every required field has a value in the displayed unit system.

☐  Boundary elevations and schedules match the approved source data.

☐  Conduit materials, shape assumptions, diameter, celerity, and friction have been reviewed.

☐  Surge tank connection and cross-section data have been reviewed.

☐  Turbine characteristics, operation mode, TOFF where applicable, and gate schedule have been verified.

☐  DTCOMP, DTOUT, TMAX, and ACCUTEST settings match the analysis plan.

☐  The Courant condition (celerity × DTCOMP / conduit length ≤ 1.0) has been checked.

☐  Output requests have been explicitly reviewed under Current Requests — HISTORY.

☐  Validation errors are resolved and warnings are understood.

☐  The generated .INP, WHAMO diagnostics, and .OUT are saved and checked.

☐  Results are reviewed by a qualified person before they are used in engineering decisions.

### Troubleshooting

| Symptom | Checks and recommended action |
| --- | --- |
| Cannot sign in | Check the account email and password, network connection, and whether the account is active. Ask the administrator for help; do not share credentials. |
| A section or button is missing | Confirm the account's role and section permissions with a Master Admin. |
| A project does not appear in Open | Confirm that it was saved to the account, that the correct user is signed in, and that the project list has refreshed. Check local backups as well. |
| A local project file will not load | Confirm the file is a valid .json project or supported .INP; keep an untouched copy and note the first displayed error. |
| A network connection is not created | Check that the correct connection tool/handles are being used and that the intended endpoints are selected. Confirm the arrow direction. |
| Validation blocks generation | Read each message, correct the indicated data/topology problem, then run validation again. Do not treat a successful bypass as proof of correctness. |
| .INP does not match the open canvas after editing | This is expected. The text edit does not change the canvas; download the edited file and import it back into the Designer. |
| .OUT generation fails | Confirm the .INP passes review, check the configured WHAMO engine and server availability, and retain the first error message. If needed, run the input with the approved local WHAMO installation. |
| .OUT opens but a graph is missing | Confirm the output contains the relevant summary/history section. History Graph is only available when history data are detected. |
| Excel rows are skipped | Preserve the exported headers, match identifiers to existing elements, and review the import summary for row-level reasons. |

When reporting a problem, record the date/time, application version, action immediately before the problem, full error text, and a minimal example file with confidential or personal information removed.

## 14. Glossary

| Term | Meaning in this manual |
| --- | --- |
| ACCUTEST | Accuracy-check setting that may be included in the generated input. |
| DTCOMP | Computation time step entered for a simulation stage. |
| DTOUT | Time interval for recording output. |
| TMAX | Simulation end time for a stage. |
| HISTORY request | A request to record selected variables for an element over the simulation history. |
| INP | WHAMO input file containing model and run configuration data. |
| OUT | WHAMO simulation output file used for reviewing results. |
| SI | International System of Units. |
| FPS | Foot-pound-second / customary English unit system as labeled in the application. |
| TCHAR | Turbine characteristic dataset selected by a turbine. |
| TOFF | Turbine load-rejection time field displayed for REJECT mode. |
| VSCHEDULE | Turbine time/gate-opening schedule referenced by the turbine operation settings. |
| WR² | Moment-of-inertia input displayed for the turbine-generator assembly. |
| D_eq | Circular equivalent diameter calculated from an entered non-circular shape reference dimension by matching cross-sectional area. |

## Before final submission

This manual is written from the current project interface and source. Before submitting it:

1.  Replace the author and organization fields on the cover.

2.  Confirm that the deployed application still displays version 1.0 and that the described controls match the deployed build.

3.  Add screenshots captured from the final authenticated application if required by the reviewer. Use sanitized example data and hide account details.

4.  Confirm turbine mode behavior and all generated file syntax against the specific WHAMO executable/version used for the project.

5.  Proofread the final Word/PDF copy and attach the project's reviewed source code, test evidence, and required signed declarations separately.

This guide does not claim that the software, model, or simulation has been independently certified or accepted by the Ministry of Jal Shakti.